CRNR

CENTER FOR REALLY NEAT RESEARCH, SYRACUSE UNIVERSITY

The CRNR houses an ongoing interdisciplinary effort to conceptually and technologically escalate the work of both the Northeast Parallel

Architectures Center (NPAC) and the Institute for Interventional Informatics (I3).

The Northeast Parallel Architectures Center (NPAC) is an advanced computing center at Syracuse University in Syracuse, New York. Directed by Geoffrey Fox, NPAC specializes in High Performance Computing and

Communications (HPCC), parallel processing, distributed computing, computational science, education, and technology transfer through the InfoMall program.

The Institute for Interventional Informatics (I3) is a non-profit organization committed to the pro-social utilization of information technologies to improve quality of life. I3 Director Dr. David Warner graduated from Loma Linda University School of Medicine in June 1995.  Also a Ph.D candidate in neurophysiology and medical informatics, Dr. Warner has been centrally concerned with human expression.  In particular, how does/may the mind  manifest itself through its body’s biological systems.  Neural, ocular and muscular tissue all generate electrical fields.  These dynamic fields are controlled willfuly through our physical and/or cognitive movements.  In medical school,  Dr. Warner noticed the potential for measuring these electro-physiological events and simultaneously using them as computer inputs: “Its a new kind of thinking-you can use your biological system as the controller.” An “interventional informatic,” then, is an informational mechanism inserted into a human system’s existent set of coping strategies which bolsters a persons power over their own life.  Appropriately, the human system with which I3’s work into physiological human-computer interfaces was initiated was the quadreplegic.  Unable to access an informatic system through conventional methods (i.e., mouse, keyboard), I3’s solution of the problem caused a paradigm shift evolving now at NPAC.

I3 has been involved in Medicine, Communication, education, wellness recretion

This document stands as an overview of this and sister projects at CRNR.

With this project individuals will be able to create new educational opportunities, employment opportunities and increase their socialization through their cultural integration into the information society.

I3 Track Record and Contacts.

Rock ‘n Roll scientists who came cared and cured

List of associated organizations

Partners for pulsar - the local cluster

Npac

Already developing the software

Ecu

Large telemedicine site to validate

Ski

Big rehab center already have been using prototypes for a year

Hit

Top human interface technology lab

Sdsu

Disabled program

Su

Bioengineering and computer science resources

Ucsd

School of medicine

Pulsars success will make web-based services accissible to the public domain of need.

PULSAR PROJECT

A Pulsating Beacon of Intelligence Illuminating the Web

Mission:

To improve quality of life through the intelligent and socially responsible applications of information technologies.

PULSAR is a superstructure for everything done at CRNR.  Beyond issues of technological development, accessibility etc., there is the urgent sense in which all technocracy is really a social process; computer system evolution is really just an evolution in how humans may express their minds to one another. Yet, the memes which have thus far sculpted technological culture are often less than socially neutral. The rather hardened cognitive and emotional patterns of corporatism severely encumber the progress we seek to achieve.  By relinquishing this approach to development, PULSAR endeavors to demonstrate through its inventions and its sharing of them that the advantages of socially integrative applications dwarf the localized and ephemeral bloat of owning a new operating system, for example.

Off-the-shelf technology is a fundamental aspect of the PULSAR philosophy.  This massively reduces costs for end-user.

How will we propagate the memes

Conferences

Siggraph launch

Comdex

Ces

Mmvr5  big roll out demo/stunt event

Imagina  international pulse

Big presence at e3

Still in its infancy, PULSAR supports five works in progress:

- Neat Tools

NeatTools Overview

NeatTools is a visual programming language that creates programs for data collection, gesture recognition, control of external devices, virtual world control, and perceptual modulation. Some of the functionality is ported from Neat Software developed for Dr. Dave Warner at the Institute for Interventional Informatics from 1993 to 1995.

The visual interface is similar to AVS. Different modules are selected, placed in a work area, and connected by lines. The connections indicate a data flow from one module to another. The line color indicates the type of data flowing.

The software consists of JavaScript, Java, and native C code. JavaScript is used for buttons and forms. Java is used for the AVS-like graphical user interface, and for JavaScript/Java/C code generation. Native C code is used for accessing hardware devices, and to bypass Java/Netscape security checks.

The NeatTools environment is extensible. New modules can be added by running tools which specifycharacteristics of the module. Code generators then provide all code required for fitting the module into the environment.

- Smart Desk

- Pulsar

- Grok Box

Evaluating the approach

Drive the need for higher through put

characterized by an ever increasing array of heterogeneous sensors and sensing modalities. This mixture of potentially vital but mostly obscure data, in it’s native form, exceeds the human limits of  integrative sensibility. This problem has been, and is being, addressed by an equally heterogeneous array of methods ranging from new AI “tricks” to graphical techniques and virtual reality interface systems. These approaches have met with varying levels of success.

Observation:

While reviewing the current projects dealing with this problem it became apparent that most solutions, or attempted solutions, attempted to solve the problem by focusing on the technology.  The projects that did address the human factors aspects of the problem appeared to be an attempt to develop better “Reality Renderers”.

Proposal:

We propose to take an approach that appears to have great potential is to aid in the overall solution of this “sensor/sensory” overload. That  is to develop an interactive environment that explores new ways to render complex information to the user by optimizing the interface system to match the human nervous systems ability to transduce-transmit and render to consciousness the necessary information. Such a system will be based on the human user’s neural information processing which directly supports perception.  A perceptualization environment could be built which optimizes the humans ability to discriminate and iterative refine emergent patterns from any variety of sensor data.

Description of the system:

The perceptualization environment, “GROK-BOX” will integrate several vital components of an interactive information environment.  Key elements include multi-sensory rendering systems, advanced human input devices and an array of computational techniques which transform the diverse data types into perceptible patterns that enhance human capacity to perceive meaningful signals in the” sea of noise”. A comprehensive set of visual, aural, tactile, proprioceptive, somatosensory and olfactory rendering devices will be integrated into the system to give the user an integrative experiential interaction with the complex data types. The system will also integrate several unique input systems which allow the user to have a multiplicity of interaction options in which to communicate the perceived significance to the system for further enhancement.

Really Cool Stuff:

The Grok-Box will be a  vehicle to explore a wide varityt of natural and un-natural perceptualization techniques.

Unifying each of these projects technically and socially is the backbone of the WWW.  Both academic and private scetor research and Web hub for intelligent systems and disabled resources.

As the work began with quadreplegic users, the interface designs are currently tailored to the abilitities of such users.  The aim of the research is ultimately the idea that CRNR is conceiving of and developing systems whereby a machine becomes able to recognize any user’s ability profile and reconfigure its interface appropriateley.  Data fusion of sensor data with user interaction parameters allow meaningful corelations of performance modalitites.  Cognitively induce modulation of some of the natural rhythms.  Consistent in an individual but different between individuals.  Psycho-social-behavioral nature of individuals factor into assessment of their cognitive function. Interface is mapping onto the body.   Here we are speaking about AI powered machine adaptive resonance.

Technologies as extenders and adapters of capability, and ways of thought and communication that foster inclusiveness.

Goals:

Create a Web-based system that enables a disabled person to: do art, play music, navigate around the Web, communicate, control telerobots, experience telepresence and ultimately do real valuable work.  To make accessible intelligent systems of interactive information technology.  Develop a Web services network for disabled individuals: social impact justifies investment by enabling the greatest number of disabled individuals to expressively interact with the world.

In addition, we hope it may be generic enough to act as a standard for other similar applications.  Smartdesk is one such generic application.

********************************************************************

Neat Tools is the working ideal of its Predecessor, Neat Software.  Neat Tools succeeds in being the first Web-based design of the gesture control software began by Neat.  While a medical and Ph.D student Dr. Warner’s research into technologies applied to medicine he and his collegues conceived of and began development of what was called the “neuro-rehabilitation work station.”  Because the research was conducted in a medical context the original thought was clinically centered.  With Pulsar, the goal of making the technology generically applicable for all who might benefit rather than specific cases is quickly being achieved. In order to calibrate TNG 1 data with a graphical rendering Neat software yields

both an EEG like trace as well as binary gesture state vectors. The incoming signals are thus transformed into a visual portrayal of user activity. Margins of user physiological signal output are set accordingly.

Assessing and accordingly augmenting human performance is the principle of what we’re doing.  The purpose is to develop and apply Enabling technologies.  Very importantly, this includes human-computer interface technologies for medicine.  Origin:  quantitative assessment of neurological disorders with glove, EEG and MRI.  Wellness paradigm from the positive result of improved quality of life resulting in reduced health care costs.

- Rehabilitation:  application of technologies to the needs of the [severely] disabled.  In the context of clinical neurology this means quantitative assessment etc. yielding more potent and efficient treatments.  In the context of the disabled user it refers to advanced sensors and input devices (e.g., emg electrodes) which empower and thus improve quality of life-the central philosophical motivation CRNR.

- Education: VR systems and interactive multimedia enhance the presentation and manipulation of information.

- Medical Informatics:  VR systems and local-and wide area networks can help physicicans acquire, access and manipulate data

- Basic Medical Research: same as part one of rehab

interface (hardware/software0

data analysis

expert systems (machine resident intelligence)

In rehab

This technology if properly applied massively enhances the goals of rehabilitative medicine: measurement of improve the measurement of human performance and the assessment of therapeutic efficacy. Diagnostics and measurement. This work can also be generalized to multiple health-care markets.  As the model the NRW integrates multiple data acquisition devices, interface technologies, advanced analytical techniques, and multi-sensory rendering capabilities.  Emphasis is placed on machine-resident intelligence embedded at several levels:  An AI system like “Angel” neural networks to moniter patient/therapist interaction, identify data trends and patterns and once again adapt the system to the user based on all of the above. For example, linking input device data with the patient’s records and specifying sensor-specific data formats and structures.  The system thus adapts (adaptive resonance).  The information which emerges in such interaction will be available to participating physicians, therapists, patients and scientists.

Very important from a rehabiliative point of view is motivation.  Users are psychologically engaged by the ongoing dynamic and changing feedback they get from the computer; this induces firther participation etc.

To go forward we must identify and aquire rapid prototyping tools (code generators, graphical user interface libraries, and general case tools) and data bases. , our efforts to make these transfers permanent have been impeded by the lack of standard platforms, interfaces, inaccessible file formats, industrial inertia geared toward building bombs and missles…

- creation of a development env

- modular design( for dist processing and adaptability)

- Integration of several data_input devices into one platform within a common interface protocol.

- machine resident intelligence

USE DIAGRAM IN VR SPECIAL REPORT WINDOW TO CLOSE

integration of a variety of input devices into a singly system (unified platform)

data fusion (give some historical examples etc eeg/mri) allows for meaningful correlations across various performance modalities (examples from Eyal) “The devices and their hardware boards connect to an external module, and a high speed bus will route the data both to a central multi-tasking server and to the rendering subsystem for immediate feedback.  The server should be intelligent enough to automatically implement a custom configuration of input device parameters, interface functionality, and relevant records based on the device(s) connected and the identity of the operator(s) and patient(s) currently at the system  Morph to serve!  User specific real time tayloring prior to use.”

Record security The maintenance of medical record integrity is a significant issue.  Such integrity is achieved through security protocols, standardized data formats, error handling, and semi-automated database archiving. The data management subsystem tasks also include linking the device data with the patient record and specifying sensor-specific data formats and structures.

Adaptive resonance to optimise performance.

Adaptable on line help with text, graphics, and animation.

Data analysis subsystem Given that the data acquisition subsystem can detect these changes, the data analysis subsystem is designed to enhance them in ways that may then be rendered to optimize the operator's sensory modalities

Five classes of user

- Therapists (most extensive users of all functions)

- Technicians (set up/aquisition)

- Physicians (analysis)

- Researchers (data analysis/general as needed)

- Patients (biocybernetic interaction)

The NRW will implement multi-sensory rendering by combining recently developed 3D sound and tactile feedback systems with advanced visualization technologies.  The NRW will enhance perception of complex relationships by integrating visual, binaural, and tactile modalities.  The rendering subsystem has a near real-time biofeedback mode for use in a therapeutic paradigm and a data perceptualization mode for use in an analytical paradigm.  Outputs from sensing devices and analytical operations are parsed and routed to the combination of rendering modalities best suited to render that information.

The goals of the NRW are twofold; 1) to provide an open hardware platform and modular infrastructure which will expedite the implementation of new technologies into the clinic, 2) to augment clinical therapy with new methods of interaction and analysis.  Success should result in providing neurorehabilitation, and the medical community in general, with a powerful tool for characterizing the complex nature of normal and impaired human performance.  Gross and subtle changes in function will be monitered by the data acquisition devices.

At the moment, NEAT must be calibrated by the user.  Calibration sensitizes the computer to the movements of a specific user.  The goal here, and one which permeates all the projects of Pulsar is that such calibrations will be handled by the “artificially intelligent” computer.  Hence, the user needs only to put on the interface and in a short period of time the computer will have adjusted itself to its user.     This effectively sets margins within the computer to

NEAT TOOLS

Design and Implementation of an Expressional Interface with Sensor Devices and Web-Based Software.

Neat Tools is the first instance of a universal platform application of a disabled user technology like NEAT.  Neat Tools began as a refinement of an older version of gesture recognition software called BEC (bio-enviornmental software).  Conceived and developed by I3 in 1992, BEC was the first software developed to allow facial EMG signals to become computer commands. In order to render Ashley's detected facial muscular signals to a screen, BIOMUSE and TNG 1 would require software to communicate with the primary computational systemthe 386 PC. I3 programmer, Jo Johansen, first wrote Bio-Environmental Control (BEC) software to establish the connection between gesture EMG signals and specific computer responses. These would then control the behavior of specific graphical software a user actually interacts with. Neat Software, also written by Johansen, eventually substituted BEC and continues to evolve as the primary hardware to software communication for I3. In order to calibrate TNG 1 data with a graphical rendering Neat software yields both an EEG like trace as well as binary gesture state vectors. The incoming signals are thus transformed into a visual portrayal of user activity. Margins of user physiological signal output are set accordingly.  In this way the gesture input is calibrated to control a video game character (e.g., Pac Man, Super Mario Bros) or a remote control vehicle [photos].

One of the earliest goals for I3 was Integration of several data input devices into one platform within a common interface protocol.  Neat Tools accomplishes this goal (initial hardware for reconfiguring human-computer interface).  Written in Java, Neat Tools is a full-fledged Web platformed piece.  The original goal of the NRW is finally coming into fruition.  THNG 2, as a multiple channel signal transducer, provides for several types of input which all come together:  data collection, gesture recognition, control of external devices, virtual world control, and perceptual modulation.

Disabled user specific input mechanisms (i.e., sensors)

Development of an expressional interface using Neat software and input devices etc. NeatTools Overview

NeatTools is a visual programming language that creates programs for data collection, gesture recognition, control of external devices, virtual world control, and perceptual modulation. Some of the functionality is ported from Neat Software developed for Dr. Dave Warner at the Institute for Interventional Informatics from 1993 to 1995.

The visual interface is similar to AVS. Different modules are selected, placed in a work area, and connected by lines. The connections indicate a data flow from one module to another. The line color indicates the type of data flowing.

The software consists of JavaScript, Java, and native C code. JavaScript is used for buttons and forms. Java is used for the AVS-like graphical user interface, and for JavaScript/Java/C code generation. Native C code is used for accessing hardware devices, and to bypass Java/Netscape security checks.

The NeatTools environment is extensible. New modules can be added by running tools which specifycharacteristics of the module. Code generators then provide all code required for fitting the module into the environment.

Pressure, light and magnetic sensors

There are two main types of hardware designed in association with the

Neat software: one measuring

voltage specialized for myoelectric signals (Thing1,TNG1) and the other

measuring variable resistance from sensors (Thing2,TNG2).

This hardware was designed to be versatile so as to accept a wide array

of

sensory inputs and cost efficient so as to be easy to use and easy to

make. The software allows for easy manageability of data coming in from

various input devices. The incoming signals are transformed into a visual

portrayal of the user responses. This feature is useful insofar that the

user can correlate his motions to the changes in his cyber environment.

The input devices used with NeatTools are external devices that are

modified specifically for the

the user so as to employ whatever controllable actions he can make. Such

input devices vary from

commercial joysticks to the "Thing" devices, which were developed for

NeatTools application. The Thing 1

(TNG1) device, for example, is a 4 channel EMG signal detector, developed

at the Institute

for Interventional Informatics  . TNG1 utilizes the

mechanisms of muscle contraction by the changes in the potential

difference of the muscle tissues.

These potential differences can

be picked up by the EMG electrodes which are then connected to TNG1 and

NeatTools. In turn, up to four

different channels can be initiated by NeatTools to give a visual display

of the muscle movements in the

user. These channels can then be converted by the computer into commands

for a set number of rules, a

program, and/or communication via text and possible audio generation.

In the case of Eyal Sherman, the task of increasing the number of

channels increases in

difficulty not so much due to any limitations of the NeatTools program,

but rather the limitations of

Eyal himself. The workable area of the user is focused on the facial

gestures which Eyal has

sole control over. The method of exploiting the few accesible biological

channels

that he has is centered then, to

the available input devices that can be made around and on his facial

muscles so as to obtain input

data. Such devices are discussed belo

Eyal Sherman

Currently, the efforts of the project have been focus on the case of Eyal

Sherman , a 14 year old quadraplegic who is able to control only

his facial muscles. The preliminary

work was done with electromyography (EMG) to sense potential differences

resulting from

various muscle movements. Previously, EMG has been successfully

implemented to obtain signals on the computer and to navigate a remote

controlled vehicle, simply by making facial gestures.

In the course of this project, various other sensors were used including

pressure, light and magnetic sensors, all of which will be described and

compared in later sections

Eyal began with EMG

usable channels of input (e.g., cheeks, forhead, eyes, neocortex etc)

Neat Tools are a Java-based graphical enviornment for processing data coleected from various input devices.  Currently up to 16 channels can be designated to various input output mechanisms on NTs and such advantageous displays include the multichannel oscilloscope-type array which can be used to train the user to activate particular channels (?????)

CheeksàEMGàTHNG 1àNeat Tools

Sensors:

- Self adhesive electromyographic electrodes

Cadmium-Sulfide photocells Cadmium-Sulfide photocells

Cadmium-sulfide (CdS) photocells (Radio Shack cat.  276-1657) were used

with 22K ohm resistors to make photosensors which decreased resistivity (increased conductance) with an increase of light on the

photocell. These sensors were used with TNG2, measuring voltage, by

positioning the photocells on Eyals face.

- Hall Effect Transducer (HET) MagneticBy using a Hall Effect Tansducer (HET) which detects changes in a magnetic field,

facial motion could be recognized by placiing a rare earth magnet on the

appropriate facial area(s)muscle and noting movement with a HET probe . This sensor are very  sensitive to minute changes in the magnetic field and were easily adapted to the NeatTools interface, again by TNG2.

- Velostat By utilizing Velostat, which is an antistatic plastic packaging material that changes resistance with changes in pressure, signals from pressure exerted by the chin were exploited via TNG2. Sandwiching Velostat between two layers of copper gave a resistance differential when the user opened his jaw thereby applying pressure to the chinstrap. This model needs further modification in strapping it securely to the head and positioning it firmly onto one location on the chin.

Design of Web-Based NeatTools

NeatTools software is a Web-based interface system which

augments our ability to create, express, retrieve, analyze, process,

communicate, and experience information in an interactive mode via

interface devices (see above), it senses

bioelectric signals from eye, muscle and brain activity

as input and render information in ways that take advantage of

psychophysiologic signal processing of the human nervous

system . We developed and

integrated the NeatText module where the user can interface with the

outside

world with the use of text.

The NeatText module adds an easy and fast way to communicate through the

computer.

The model implemented in this module uses a triangular-based geometry

of the alphabet as seen in figure  .

htb

tri.ps

.9

A Text Triangle

F:tri

The NeatTools module allows the disabled

individual to use facial movements to select letters of the alphabet to

form

words. The number of times the user has to make a facial gesture to

select a

letter of is three. This is the reason behind the

triangle; we want the minimum number of gesture movements.

Within the one large triangle, there are three

smaller triangles with another set of three smaller triangles in each

of the three larger

triangles which consist of letters.

To get to a particular letter, the user chooses one

of three directions and simplifies the big triangle to the smaller

triangle

that the user has chosen. The user

continues to simplify the triangles until he has reached the letter he

desires. This model is very efficient; three selections in any direction

by

the user can get a specific letter. In the case

of Eyal, the sensor frame will give these three signals allowing him

to

select letters and form complete phrases more quickly than ever

before. This idea of text generation is a way to communicate without

having to use a physical keyboard input.

Two design algorithms were taken into consideration for this problem.

First algorithm was a point value system for

each movement. The initial signal to the left had a point value of 1,

right

had a point value of 10, or top had a point value of 100.

Second signal multiply by 10 and the last signal multiply by 100.

The letter would be the

one with the matching point value, but conflict in similar point values

occurred. Second algorithm that was implemented was

to divide the triangle up into

three distinct levels: TOP = level 1, RIGHT = level 2, and LEFT = level 3.

Then we take each of the three levels and break it down into three more

levels. From the TOP triangle, we are at level 1. Each triangle in the

TOP triangle will be numbered: TOP, RIGHT : 12 ; TOP, LEFT : 13 ; TOP,

TOP : 11. The same type of numbering was done for RIGHT and LEFT ( level

2 and level 3 ). The user can to choose a letter now. All the cases of

third choice were taken from all the options of the first two choices.

The NeatText module that was created was implemented in Java and

integrated into the NeatTools environment.

One great feature of Java is platform independence, which means that your

program can run on any system from PC to a workstation, but

the trade off is speed.

Our first notation of how slow Java is in the usage of fourteen breakdowns

of the triangular image to display the users selection

in the triangular-model.

This idea slowed down the process

because the user would have made a selection and the image was still

trying

to download. Our next alternative was to use a Java feature called "crop

filtering with MediaTracking" which used one triangle image. Every time

the

user selected a particular region in the triangle, that region would be

highlighted.

This idea increased the

performance of the algorithm, but not to our satisfaction. The

alternative that we implemented and that does not

slow the process down as much was to black out the other two triangles

that were not chosen. The reason why this alternative was faster is

because we did not have to create an extra object.

Another factor that has contributed to the slow down of the algorithm

was the continuous incoming signals from the serial port.

This continuous stream of input signals causes data

congestion between the modules.

Three techniques were used: first, we cleared the buffer of the

previous input signals. Although minimized, collision and interference

still

impeded the interpretation of signals. Next, we reduced collisions by

periodically dropping redundant values from the sigan;. Finally, a data

delay was introduced to control the number of imcoming signals being

processed.

The NeatText module opens up numerous possibilities for the future.

The simple triangle model can be extended into a pyramid-model to consist

of commin phrases and reserved key words for URL. Word processing

applications like MS-Word and Notepad are future extensions of the text

area in the NeatText module. Through text on the screen, it is possible

to write URL addresses which extends to Netscape to navigate through the

World Wide Web (WWW).

Computer Data Rendering

After establishing sensory inputs and a computer interface, the final

aspect of the interface cycle is the computer output, or rendering, that

the user can recognize as changes in his environment.  By correlating

these changes with the changes in sensory input as provided by the user,

the user can identify what input causes what changes; this completes

the interface cycle.

Computer Interface Results

This part of the project was intended mainly to design a specialized,

user-friendly interface for disabled people.  The user interface is based

on a set of frames and Java applets that help a user to

communicate easily with the World Wide Web.

One of the frames provides an option to enter a new URL into a

text editor or choose one from the bookmark list provided the context of

that Web page will be displayed in the target frame, that is located

underneath the first frame.

The NeatTools application is displayed in the frame on the bottom of

the Web page.  It allows the user to set it up, to load a saved file,

and to run it.  When NeatTools runs, a pop-up window of the

NeatText comes up and the user can type text in the window provided.

On the left side there is a menu frame, which  contains the items

that can be added.  One of them is a "Pointer Icon" and the other one is

"Telnet via Nova".

Pointer Icon is a prototype for a software control device over

the mouse pointer, which will be using alternative input devices, such as

TNG1 and TNG2, form Institute of Interventional Informatics.  The time

Pointer Icon

prototype exists in a Java applet version.

htb

reu-annab-pointerwin.ps

.8

The "Pointer Icon" Java applet shows the prototype of the control

devise over the mouse pointer.

F:pointerwin

As shown in Figure   Pointer Icon consists of a

circle and a ball.  The five buttons controlling the movement of the

Pointer

icon are at the top of the applet:  Stop, Left, Right, Move, and Click.

One click on the Left button

makes the ball rotate around the circle in the counterclockwise

direction.  Clicking on the Right button makes the ball rotate around the

circle in the clockwise direction.  The Move button allows the whole

circle to move in the direction of the ball, as shown in the

Figure  . The movement of the ball and the circle is

stopped by clicking the Stop button.

htb

reu-annab-pointerclr.ps

.4

The big solid arrow represents the mouse pointer.  The small

solid

arrows show the direction in which the ball could rotate around

the

circle.  The doted line shows the direction of the movement of the

Pointer Icon.

F:pointerclr

The Click button will be implemented so that the inputs to the buttons

will be accessed through TNG1 or TNG2 via the serial port of a PC, in

order to control the

movements of the circle and the ball.  The Pointer Icon will send

the x and y coordinates to the mouse pointer which will be moving with

the circle in the direction of the ball, as shown in

Figure  .  That is where the implementation of the

Click button will be implemented.

The second item in the menu frame is "Telnet via Nova".  This

frame provides easy access to Telnet.  Without closing the Web

browser it provides a new Telnet window.  Actual use of this function for

Eyal will be challenging.

The Pulsar Project

We are also in the process of designing a Web page for the Pulsar

project.  This page will provide a Web-based interface systems and

free Web services for supporting accessible interfaces for disabled

people.  The goal of the Pulsar project is to develop a Web service

which will be a clearing house for tools for disabled individuals.  We

will provide a database of empowering software which is to be available to

download for free and a list of off-the-shelf devices for the

disabled. There will be a list of local support groups and resources

provided . Pulsar will also develop a forum where users can exchange

information on methods and techniques which empower the users

to become employed members of the information society.

Acknowledgements

The following people have helped with this project in various ways:

Johannes Johannsen, scribe of NeatTools; Alvin Leung from NPAC, Lou

Buda and Sam Sampere from Physics, all the students from the REU

program. The Sherman Family and especially Eyal Sherman for all their

hard work and determination. Thanks for making the summer exciting,

educational and fun.

reu-sshah-bib

Part of the informatic solution is user application set up time.  Currently it takes around 30 minutes or more to set up the user sensor peices.  There are also other problems which emerge from the current sensor technology such as slippage, inconsistent performance etc.  Such problems will be solved by our researhc into miniaturizing both sensors and holders.  In addition we are also woking on more sophisticated signal processing.  At the moment the sensors are recognizing simple binary threshold detectin for each input channel (i.e., chin, cheek, forhead movements etc).  Accessing analog and timing information would allow many more channels of information to come in from each sensor.  This would augment already existing abilities for mouse-cursor control; Web surfing; rapid typing; speech generation from typed text simplicity conveneience, modularity, standardization so users and their families can easily manage these low-cost technologies (Pulsar Philosophy!).

initially this began with self adhesive electrodes.  Our goal now is to engineer non-adheisve/invasive sensors.

Biocar:  parallel port of the PC and a modified Radio Shack (Tyco) hand controller (acknowledge Patrick Keller, Markus Schmidt, Solomo Maturner)

pacman, modified sega controller, Tetris for java version text triangle.